# Fuhrlander Research Log

FL2500 SCADA dataset  |  causal discovery pipeline  |  fault detection

# The Paper

Nature Scientific Data 2024 - a dataset paper. The contribution is the data, not a model. Five Fuhrlander FL2500 turbines (WT80-84) in one wind farm, three years of SCADA at 5-minute resolution. WT84 had a real gearbox bearing failure during collection, giving the dataset genuine ground truth for fault detection research.

| 5 | 312 | 1,248 | 5 min |
| --- | --- | --- | --- |
| turbines WT80-84 | analog sensors | signals (x4 stats) | raw resolution |

Each sensor stored as avg / max / min / sdv per 5-minute window. Format: one .json.bz2 per turbine + wind_plant_data.json (alarm dictionary, 64 Transmission alarm types). WT84 data runs 2012-01-01 to 2014-12-08 with 594 transmission alarms in 2014 alone - the failure cluster.

# What the R + MATLAB Codebase Does

## R - examples.R + turbine_data.R

Loads one turbine JSON.bz2, filters to Transmission alarm IDs, resamples 5-min to 1-hour using filtered_3sdv_mean (discard readings outside mean +/- 3 sigma per block then average), saves to .RData. Pure data prep - no model.

## MATLAB - feature_selection.m (Two-Phase RF)

Runs on combined WT80-83 training data (first 50% of each turbine). Before the RF runs, each hourly row is stamped with a binary alarm_active flag: 1 if any Transmission alarm was firing during that hour, 0 otherwise.

How alarm_active is built (concrete example):

Raw alarm log (Transmission alarms only):
  id=1369  MGB Temp OilSump < SL  2014-02-02 09:34  ->  2014-02-02 09:35
  id=964   MGB FuseTrip OilPump   2014-02-03 14:41  ->  2014-02-03 14:42

Stamped onto hourly rows:
  2014-01-15 08:00  bearing_temp=38.2  wind=9.1  ...  alarm_active=0
  2014-02-02 09:00  bearing_temp=54.1  wind=8.8  ...  alarm_active=1  <- alarm firing
  2014-02-03 14:00  bearing_temp=57.3  wind=9.3  ...  alarm_active=1  <- alarm firing
  2014-04-10 11:00  bearing_temp=39.1  wind=7.2  ...  alarm_active=0

Phase 1 - find the best target sensor (Y_VAR):

RF is trained: given all 312 sensor readings in an hour, predict alarm_active (0 or 1). After training, the sensor whose shuffled values hurt predictions most = most important. Winner: wtrm_avg_TrmTmp_GbxBrg152 (gearbox high-speed shaft bearing temperature). Makes physical sense - the bearing runs hot exactly when alarms fire. This sensor becomes Y_VAR. Alarms are never used again after this point.

Phase 2 - find the best input sensors (X_VARS):

RF is trained on the remaining 311 sensors predicting the numeric value of bearing temperature (not 0/1 anymore - a real temperature number like 38.2, 54.1). Top 10 sensors by importance become X_VARS. Things like ambient temperature, generator winding temp, rotor speed - sensors that physically drive how hot the bearing runs.

## MATLAB - elm_classifier.m + example_elm.m

Trains an ELM regressor on (X_VARS to Y_VAR). Random input weights W and biases b fixed at init, sigmoid hidden layer, output weights B solved via pseudoinverse. Tries K = 1 to 150 hidden neurons, keeps best RMSE. Evaluates on WT80-83 second 50%. WT84 is never touched.

## Ultimate output

"Given these 10 sensors, bearing temperature can be predicted with low normalised RMSE." Proof of concept only - no early-warning timeline, no fault flag, no WT84 analysis. The implied next step (train on healthy data, watch WT84 residual grow before failure) is left open for the research community.

Does NOT predict failure

Does NOT run on WT84

DOES prove the selected features are useful

DOES provide a reusable two-phase RF selection method

# Session Log

## 2026-08-06

Explored dataset:JSON structure, alarm dictionary, WT84 failure cluster (594 transmission alarms in 2014), analog data date ranges per turbine

Understood R+MATLAB fully: R just resamples, MATLAB proves bearing temp is predictable (RMSE only), WT84 never touched

Built 01_data_prep.ipynb: Auto-detects file format (csv / xlsx / json / json.bz2), 9-step clean pipeline (impute, IQR smooth, drift removal, z-score), ECG plots with red time-gap bands, sanity check table

Built python_pipeline.ipynb (FUHRLANDER GITHUB REPO): Full Python rip-off of R+MATLAB. Loads all turbines, resamples with filtered_3sdv_mean, two-phase RF, ELM (pseudoinverse, tries K 25-150). Applies to full WT84 timeline - 3-panel anomaly chart with failure date marker

## Tomorrow

Build 02_feature_selection.ipynb: Variance filter, correlation drop, two-phase RF (Phase 1 finds best target from alarms, Phase 2 finds best predictors of that target). Auto-skips Phase 1 if target already set

Build 03_pre_causal.ipynb: ADF stationarity, kurtosis, QQ plots, ACF, assumption summary table, time bucketing to CSVs for causal discovery

Run python_pipeline.ipynb - check if anomaly score rises before June 2014 failure marker

Feed step3 output into LiNGAM to get the causal graph